SOURCE REFERENCE 00

시스템 아키텍처 및 소스 인덱스

전체 런타임 데이터 흐름, 스레드 경계, 구성·프레임 계약, 문서 범위와 파일별 찾아보기

| 프로젝트 | FMCW LiDAR v0.2.0 |
| --- | --- |
| 기준 소스 | 현재 worktree의 active build source |
| 문서 기준일 | 2026-08-31 |
| 문서 목적 | 함수 입출력·연산·호출 관계·출력/부작용 추적 |

# 문서 사용 범위

포함: CMake active source, 공용 header, Windows/Jetson 앱, MCU custom/generated Core, ROS package, 배포·도구·테스트 코드.

제외: STM32 HAL/CMSIS vendor 함수 본문, legacy/OpenCV/SDK vendor code, build·package 산출물. 이들은 연결 경계와 의존성만 기록한다.

읽는 법: 각 함수 항목은 선언 위치, 입력, 내부 연산, 호출/호출자, 반환·상태 변경, 오류·동시성 특성을 순서대로 보여준다.

# 1. 런타임 구성

Windows와 Jetson은 동일한 fmcw_core 및 Qt 공용 계층을 사용한다. 플랫폼 차이는 실행 entry point, AlazarTech/CUDA 라이브러리 탐색, serial 장치 이름과 package script에 한정된다. CPU FFTW와 CUDA cuFFT는 동일한 chirp segmentation·window/DC·peak·거리·XYZIV 계약을 구현하며 Jetson 제품 빌드는 CUDA를 요구한다.

# 2. 주 데이터 흐름

| 단계 | 주체 | 입력·연산 | 출력/다음 단계 |
| --- | --- | --- | --- |
| 1 | Qt MainWindow | operator 설정·start/stop·표시 선택 | ApplicationController |
| 2 | ApplicationController | 설정 revision 고정, service/장치 lifecycle | AcquisitionSession |
| 3 | AcquisitionSession | EDFA 준비 → digitizer arm → MCU trigger enable | ContinuousAcquisitionWorker |
| 4 | Alazar/Replay/Simulator | RawFrameBatch: records × full-period samples | ProcessingService |
| 5 | CPU FFTW / CUDA cuFFT | UP/DOWN 분할 → FFT → threshold peak → 거리·속도 | PointCloudFrame XYZIV |
| 6 | Frame completion | B-scan/3D snapshot, 저장, UDP, 객체 검출 | Qt / file / ROS / CenterPoint |

# 3. 시작·정지 순서

Apply Setup | ConfigManager load/apply → ConfigValidator → runtime adapter configure. scanner 관련 fingerprint가 바뀐 경우에만 MCU waveform loaded 상태를 무효화한다.

Connect | RuntimeAdapterFactory가 source에 맞는 Alazar/Replay/Simulator와 MCU/EDFA adapter를 만들고 각 장치에 연결한다.

START | storage/UDP/processing 준비 → acquisition worker 준비 → digitizer DMA arm → MCU scan/B-trigger enable. EDFA Controlled는 arm 전에 출력이 준비된다.

Run | DMA batch는 bounded queue로 이동한다. GUI는 선택 A-scan snapshot만 낮은 rate로 복사하고 B-scan/3D는 frame 완료 시 교체한다.

STOP/E-STOP | MCU trigger 제거 → digitizer abort/stop → acquisition worker join → queued processing 정리 → processing/UDP/storage finalize → EDFA off.

# 4. 데이터 계약

| 계약 | 내용 |
| --- | --- |
| RawFrame | 단일 channel의 full-period ADC sample, UP/DOWN segment, config revision, optical state, scan position. |
| RawFrameBatch | 한 DMA buffer의 record 묶음. metadata record_count/record_length/sequence가 각 RawFrame metadata와 일치해야 한다. |
| ProcessedMeasurement | UP/DOWN peak bin(분수 bin 포함), magnitude, 거리, velocity. threshold 미달은 NaN으로 표현한다. |
| PointCloudPoint | 일반 LiDAR 좌표계 X=forward, Y=left, Z=up와 intensity, radial velocity를 갖는 XYZIV. |
| PointCloudFrame | scanner의 y_line_count가 완료된 한 frame 단위 point 집합. GUI·저장·UDP·검출은 완료 frame만 소비한다. |

# 5. 실행 스레드와 queue

| 실행 경계 | 책임 |
| --- | --- |
| Qt GUI | 설정/상태/plot paint. DMA 또는 FFT를 직접 수행하지 않는다. |
| Acquisition | AlazarWaitAsyncBufferComplete 또는 replay/simulator 대기, batch metadata/trajectory stamping, processing queue push. |
| Processing | CPU/GPU batch 처리, frame assembly, snapshot publish, storage/UDP/detection fan-out. |
| Storage | raw/point-cloud file write와 주기 flush. 수집 thread의 disk I/O를 제거한다. |
| UDP | 완료 point-cloud frame fragment 송신. bounded queue 정책으로 수집을 막지 않는다. |
| Detection | 최신 완료 frame을 CenterPoint backend에 넣고 detection snapshot을 frame id와 함께 publish한다. |

# 6. 소스 범위와 외부 경계

이 참조서는 프로젝트가 소유한 active source를 상세 대상으로 한다. STM32 HAL/CMSIS, ATS-SDK, Qt, FFTW, CUDA/cuFFT, cuDNN 및 외부 CenterPoint runtime은 호출 API와 빌드 연결만 기록한다. legacy는 설계 근거 비교용으로 보존하되 현행 함수 호출 그래프에는 포함하지 않는다.

# 7. 전체 소스 인덱스

| 파일 | 언어 | 함수 | 수록 문서 |
| --- | --- | --- | --- |
| CMakeLists.txt | Text | 0 | 07_ROS_Tools_Deployment_Tests |
| CMakePresets.json | JSON | 0 | 07_ROS_Tools_Deployment_Tests |
| config/calibration/default.yaml | YAML | 0 | 07_ROS_Tools_Deployment_Tests |
| config/default.yaml | YAML | 0 | 07_ROS_Tools_Deployment_Tests |
| config/jetson.yaml | YAML | 0 | 07_ROS_Tools_Deployment_Tests |
| config/profiles/ats9371_200hz_simulator.yaml | YAML | 0 | 07_ROS_Tools_Deployment_Tests |
| config/profiles/lab_simulator.yaml | YAML | 0 | 07_ROS_Tools_Deployment_Tests |
| config/windows.yaml | YAML | 0 | 07_ROS_Tools_Deployment_Tests |
| deploy/jetson/build.sh | Shell | 3 | 07_ROS_Tools_Deployment_Tests |
| deploy/jetson/check_dependencies.sh | Shell | 5 | 07_ROS_Tools_Deployment_Tests |
| deploy/jetson/export_source.ps1 | PowerShell | 0 | 07_ROS_Tools_Deployment_Tests |
| deploy/jetson/jetson.env | Environment Config | 0 | 07_ROS_Tools_Deployment_Tests |
| deploy/jetson/package.sh | Shell | 0 | 07_ROS_Tools_Deployment_Tests |
| deploy/jetson/run.sh | Shell | 0 | 07_ROS_Tools_Deployment_Tests |
| deploy/windows/build_centerpoint.ps1 | PowerShell | 0 | 07_ROS_Tools_Deployment_Tests |
| deploy/windows/package.ps1 | PowerShell | 0 | 07_ROS_Tools_Deployment_Tests |
| Ros_project/src/CMakeLists.txt | Text | 0 | 07_ROS_Tools_Deployment_Tests |
| Ros_project/src/fmcw_lidar_rviz/CMakeLists.txt | Text | 0 | 07_ROS_Tools_Deployment_Tests |
| Ros_project/src/fmcw_lidar_rviz/include/fmcw_lidar_rviz/udp_point_protocol.hpp | C++ Header | 10 | 07_ROS_Tools_Deployment_Tests |
| Ros_project/src/fmcw_lidar_rviz/launch/udp_rviz.launch | ROS Launch XML | 0 | 07_ROS_Tools_Deployment_Tests |
| Ros_project/src/fmcw_lidar_rviz/package.xml | XML | 0 | 07_ROS_Tools_Deployment_Tests |
| Ros_project/src/fmcw_lidar_rviz/rviz/fmcw_lidar.rviz | RViz Config | 0 | 07_ROS_Tools_Deployment_Tests |
| Ros_project/src/fmcw_lidar_rviz/src/udp_point_protocol.cpp | C++ | 12 | 07_ROS_Tools_Deployment_Tests |
| Ros_project/src/fmcw_lidar_rviz/src/udp_pointcloud_receiver_node.cpp | C++ | 15 | 07_ROS_Tools_Deployment_Tests |
| Ros_project/src/fmcw_lidar_rviz/src/udp_test_sender_node.cpp | C++ | 7 | 07_ROS_Tools_Deployment_Tests |
| Ros_project/src/fmcw_lidar_rviz/test/udp_point_protocol_test.cpp | C++ | 11 | 07_ROS_Tools_Deployment_Tests |
| src/apps/common/application_controller.cpp | C++ | 67 | 05_Qt_GUI_Applications |
| src/apps/common/application_controller.h | C/C++ Header | 41 | 05_Qt_GUI_Applications |
| src/apps/common/main_window.cpp | C++ | 80 | 05_Qt_GUI_Applications |
| src/apps/common/main_window.h | C/C++ Header | 57 | 05_Qt_GUI_Applications |
| src/apps/common/plot_widgets.cpp | C++ | 35 | 05_Qt_GUI_Applications |
| src/apps/common/plot_widgets.h | C/C++ Header | 25 | 05_Qt_GUI_Applications |
| src/apps/common/point_cloud_widget.cpp | C++ | 29 | 05_Qt_GUI_Applications |
| src/apps/common/point_cloud_widget.h | C/C++ Header | 26 | 05_Qt_GUI_Applications |
| src/apps/common/replay_setup_loader.cpp | C++ | 8 | 05_Qt_GUI_Applications |
| src/apps/common/replay_setup_loader.h | C/C++ Header | 4 | 05_Qt_GUI_Applications |
| src/apps/jetson/main.cpp | C++ | 1 | 05_Qt_GUI_Applications |
| src/apps/windows/main.cpp | C++ | 1 | 05_Qt_GUI_Applications |
| src/CMakeLists.txt | Text | 0 | 07_ROS_Tools_Deployment_Tests |
| src/core/acquisition_session.cpp | C++ | 21 | 01_Core_Configuration_Acquisition |
| src/core/acquisition_session.h | C/C++ Header | 18 | 01_Core_Configuration_Acquisition |
| src/core/app_version.cpp | C++ | 2 | 01_Core_Configuration_Acquisition |
| src/core/app_version.h | C/C++ Header | 2 | 01_Core_Configuration_Acquisition |
| src/core/config_document.cpp | C++ | 25 | 01_Core_Configuration_Acquisition |
| src/core/config_document.h | C/C++ Header | 12 | 01_Core_Configuration_Acquisition |
| src/core/config_manager.cpp | C++ | 13 | 01_Core_Configuration_Acquisition |
| src/core/config_manager.h | C/C++ Header | 9 | 01_Core_Configuration_Acquisition |
| src/core/config_policy.cpp | C++ | 6 | 01_Core_Configuration_Acquisition |
| src/core/config_policy.h | C/C++ Header | 4 | 01_Core_Configuration_Acquisition |
| src/core/config_profile.cpp | C++ | 16 | 01_Core_Configuration_Acquisition |
| src/core/config_profile.h | C/C++ Header | 8 | 01_Core_Configuration_Acquisition |
| src/core/config_types.cpp | C++ | 43 | 01_Core_Configuration_Acquisition |
| src/core/config_types.h | C/C++ Header | 41 | 01_Core_Configuration_Acquisition |
| src/core/config_validation.cpp | C++ | 9 | 01_Core_Configuration_Acquisition |
| src/core/config_validation.h | C/C++ Header | 4 | 01_Core_Configuration_Acquisition |
| src/core/continuous_acquisition_worker.cpp | C++ | 7 | 01_Core_Configuration_Acquisition |
| src/core/continuous_acquisition_worker.h | C/C++ Header | 9 | 01_Core_Configuration_Acquisition |
| src/core/device_interfaces.h | C/C++ Header | 41 | 01_Core_Configuration_Acquisition |
| src/core/digitizer_capabilities.cpp | C++ | 10 | 01_Core_Configuration_Acquisition |
| src/core/digitizer_capabilities.h | C/C++ Header | 8 | 01_Core_Configuration_Acquisition |
| src/core/frame_types.h | C/C++ Header | 49 | 01_Core_Configuration_Acquisition |
| src/core/logging.cpp | C++ | 7 | 01_Core_Configuration_Acquisition |
| src/core/logging.h | C/C++ Header | 9 | 01_Core_Configuration_Acquisition |
| src/core/operation_controller.cpp | C++ | 16 | 01_Core_Configuration_Acquisition |
| src/core/operation_controller.h | C/C++ Header | 16 | 01_Core_Configuration_Acquisition |
| src/core/raw_frame_batch_pool.cpp | C++ | 4 | 01_Core_Configuration_Acquisition |
| src/core/raw_frame_batch_pool.h | C/C++ Header | 3 | 01_Core_Configuration_Acquisition |
| src/core/realtime_thread.cpp | C++ | 3 | 01_Core_Configuration_Acquisition |
| src/core/realtime_thread.h | C/C++ Header | 5 | 01_Core_Configuration_Acquisition |
| src/core/scan_trajectory.cpp | C++ | 7 | 01_Core_Configuration_Acquisition |
| src/core/scan_trajectory.h | C/C++ Header | 4 | 01_Core_Configuration_Acquisition |
| src/core/system_state.cpp | C++ | 10 | 01_Core_Configuration_Acquisition |
| src/core/system_state.h | C/C++ Header | 10 | 01_Core_Configuration_Acquisition |
| src/core/system_telemetry.h | C/C++ Header | 0 | 01_Core_Configuration_Acquisition |
| src/detection/centerpoint_cuda_compat.h | C/C++ Header | 0 | 03_Processing_Detection |
| src/detection/centerpoint_detector.cpp | C++ | 8 | 03_Processing_Detection |
| src/detection/centerpoint_detector_stub.cpp | C++ | 2 | 03_Processing_Detection |
| src/detection/centerpoint_input_builder.cpp | C++ | 4 | 03_Processing_Detection |
| src/detection/centerpoint_input_builder.h | C/C++ Header | 4 | 03_Processing_Detection |
| src/detection/detection_types.h | C/C++ Header | 1 | 03_Processing_Detection |
| src/detection/object_detection_policy.cpp | C++ | 1 | 03_Processing_Detection |
| src/detection/object_detection_policy.h | C/C++ Header | 1 | 03_Processing_Detection |
| src/detection/object_detection_service.cpp | C++ | 10 | 03_Processing_Detection |
| src/detection/object_detection_service.h | C/C++ Header | 10 | 03_Processing_Detection |
| src/detection/object_detector.cpp | C++ | 3 | 03_Processing_Detection |
| src/detection/object_detector.h | C/C++ Header | 11 | 03_Processing_Detection |
| src/detection/point_cloud_file_loader.cpp | C++ | 9 | 03_Processing_Detection |
| src/detection/point_cloud_file_loader.h | C/C++ Header | 1 | 03_Processing_Detection |
| src/drivers/alazar/alazar_digitizer.cpp | C++ | 29 | 02_Device_Drivers_Protocols |
| src/drivers/alazar/alazar_digitizer.h | C/C++ Header | 19 | 02_Device_Drivers_Protocols |
| src/drivers/alazar/alazar_sample_conversion.cpp | C++ | 1 | 02_Device_Drivers_Protocols |
| src/drivers/alazar/alazar_sample_conversion.h | C/C++ Header | 1 | 02_Device_Drivers_Protocols |
| src/drivers/edfa/edfa_protocol.cpp | C++ | 17 | 02_Device_Drivers_Protocols |
| src/drivers/edfa/edfa_protocol.h | C/C++ Header | 14 | 02_Device_Drivers_Protocols |
| src/drivers/edfa/edfa_serial_controller.cpp | C++ | 16 | 02_Device_Drivers_Protocols |
| src/drivers/edfa/edfa_serial_controller.h | C/C++ Header | 15 | 02_Device_Drivers_Protocols |
| src/drivers/mcu/mcu_protocol.cpp | C++ | 23 | 02_Device_Drivers_Protocols |
| src/drivers/mcu/mcu_protocol.h | C/C++ Header | 13 | 02_Device_Drivers_Protocols |
| src/drivers/mcu/mcu_serial_controller.cpp | C++ | 12 | 02_Device_Drivers_Protocols |
| src/drivers/mcu/mcu_serial_controller.h | C/C++ Header | 12 | 02_Device_Drivers_Protocols |
| src/drivers/replay/replay_digitizer.cpp | C++ | 12 | 02_Device_Drivers_Protocols |
| src/drivers/replay/replay_digitizer.h | C/C++ Header | 11 | 02_Device_Drivers_Protocols |
| src/drivers/runtime_adapter_factory.cpp | C++ | 1 | 02_Device_Drivers_Protocols |
| src/drivers/runtime_adapter_factory.h | C/C++ Header | 2 | 02_Device_Drivers_Protocols |
| src/drivers/serial/serial_transport.cpp | C++ | 17 | 02_Device_Drivers_Protocols |
| src/drivers/serial/serial_transport.h | C/C++ Header | 21 | 02_Device_Drivers_Protocols |
| src/drivers/simulator/fake_digitizer.cpp | C++ | 26 | 02_Device_Drivers_Protocols |
| src/drivers/simulator/fake_digitizer.h | C/C++ Header | 17 | 02_Device_Drivers_Protocols |
| src/drivers/simulator/fake_edfa.cpp | C++ | 12 | 02_Device_Drivers_Protocols |
| src/drivers/simulator/fake_edfa.h | C/C++ Header | 11 | 02_Device_Drivers_Protocols |
| src/drivers/simulator/fake_mcu.cpp | C++ | 10 | 02_Device_Drivers_Protocols |
| src/drivers/simulator/fake_mcu.h | C/C++ Header | 10 | 02_Device_Drivers_Protocols |
| src/firmware/mcu/FMCW_LiDAR_MCU/Core/Inc/ad5664.h | C/C++ Header | 1 | 06_MCU_Firmware |
| src/firmware/mcu/FMCW_LiDAR_MCU/Core/Inc/frame_player.h | C/C++ Header | 11 | 06_MCU_Firmware |
| src/firmware/mcu/FMCW_LiDAR_MCU/Core/Inc/main.h | C/C++ Header | 1 | 06_MCU_Firmware |
| src/firmware/mcu/FMCW_LiDAR_MCU/Core/Inc/mirrorcle_drv.h | C/C++ Header | 7 | 06_MCU_Firmware |
| src/firmware/mcu/FMCW_LiDAR_MCU/Core/Inc/spi24_tx.h | C/C++ Header | 6 | 06_MCU_Firmware |
| src/firmware/mcu/FMCW_LiDAR_MCU/Core/Inc/stm32h7xx_hal_conf.h | C/C++ Header | 0 | 06_MCU_Firmware |
| src/firmware/mcu/FMCW_LiDAR_MCU/Core/Inc/stm32h7xx_it.h | C/C++ Header | 10 | 06_MCU_Firmware |
| src/firmware/mcu/FMCW_LiDAR_MCU/Core/Inc/uart_cmd.h | C/C++ Header | 7 | 06_MCU_Firmware |
| src/firmware/mcu/FMCW_LiDAR_MCU/Core/Src/ad5664.c | C | 2 | 06_MCU_Firmware |
| src/firmware/mcu/FMCW_LiDAR_MCU/Core/Src/frame_player.c | C | 14 | 06_MCU_Firmware |
| src/firmware/mcu/FMCW_LiDAR_MCU/Core/Src/main.c | C | 13 | 06_MCU_Firmware |
| src/firmware/mcu/FMCW_LiDAR_MCU/Core/Src/mirrorcle_drv.c | C | 10 | 06_MCU_Firmware |
| src/firmware/mcu/FMCW_LiDAR_MCU/Core/Src/spi24_tx.c | C | 8 | 06_MCU_Firmware |
| src/firmware/mcu/FMCW_LiDAR_MCU/Core/Src/stm32h7xx_hal_msp.c | C | 8 | 06_MCU_Firmware |
| src/firmware/mcu/FMCW_LiDAR_MCU/Core/Src/stm32h7xx_it.c | C | 11 | 06_MCU_Firmware |
| src/firmware/mcu/FMCW_LiDAR_MCU/Core/Src/syscalls.c | C | 18 | 06_MCU_Firmware |
| src/firmware/mcu/FMCW_LiDAR_MCU/Core/Src/sysmem.c | C | 1 | 06_MCU_Firmware |
| src/firmware/mcu/FMCW_LiDAR_MCU/Core/Src/system_stm32h7xx.c | C | 3 | 06_MCU_Firmware |
| src/firmware/mcu/FMCW_LiDAR_MCU/Core/Src/uart_cmd.c | C | 12 | 06_MCU_Firmware |
| src/firmware/mcu/FMCW_LiDAR_MCU/Core/Startup/startup_stm32h750vbtx.s | Assembly | 0 | 06_MCU_Firmware |
| src/firmware/mcu/FMCW_LiDAR_MCU/FMCW_LiDAR_MCU.ioc | STM32CubeMX | 0 | 06_MCU_Firmware |
| src/firmware/mcu/FMCW_LiDAR_MCU/STM32H750VBTX_FLASH.ld | Linker Script | 0 | 06_MCU_Firmware |
| src/firmware/mcu/FMCW_LiDAR_MCU/STM32H750VBTX_RAM.ld | Linker Script | 0 | 06_MCU_Firmware |
| src/network/udp_point_protocol.cpp | C++ | 6 | 04_Storage_Network_Replay |
| src/network/udp_point_protocol.h | C/C++ Header | 2 | 04_Storage_Network_Replay |
| src/network/udp_sender_service.cpp | C++ | 11 | 04_Storage_Network_Replay |
| src/network/udp_sender_service.h | C/C++ Header | 8 | 04_Storage_Network_Replay |
| src/processing/cpu/fftw_backend.cpp | C++ | 10 | 03_Processing_Detection |
| src/processing/cuda/cuda_fft_backend.cu | CUDA C++ | 10 | 03_Processing_Detection |
| src/processing/cuda/cuda_module_policy.h | C/C++ Header | 1 | 03_Processing_Detection |
| src/processing/cuda/cuda_signal_pipeline.cu | CUDA C++ | 26 | 03_Processing_Detection |
| src/processing/cuda/cuda_signal_pipeline.h | C/C++ Header | 13 | 03_Processing_Detection |
| src/processing/cuda/cuda_signal_pipeline_stub.cpp | C++ | 10 | 03_Processing_Detection |
| src/processing/fft_backend.h | C/C++ Header | 5 | 03_Processing_Detection |
| src/processing/fft_backends.cpp | C++ | 13 | 03_Processing_Detection |
| src/processing/fft_backends.h | C/C++ Header | 19 | 03_Processing_Detection |
| src/processing/point_cloud_postprocessor.cpp | C++ | 15 | 03_Processing_Detection |
| src/processing/point_cloud_postprocessor.h | C/C++ Header | 9 | 03_Processing_Detection |
| src/processing/processing_service.cpp | C++ | 35 | 03_Processing_Detection |
| src/processing/processing_service.h | C/C++ Header | 19 | 03_Processing_Detection |
| src/processing/processing_snapshots.cpp | C++ | 12 | 03_Processing_Detection |
| src/processing/processing_snapshots.h | C/C++ Header | 12 | 03_Processing_Detection |
| src/processing/signal_processor.cpp | C++ | 31 | 03_Processing_Detection |
| src/processing/signal_processor.h | C/C++ Header | 17 | 03_Processing_Detection |
| src/storage/async_storage_service.cpp | C++ | 19 | 04_Storage_Network_Replay |
| src/storage/async_storage_service.h | C/C++ Header | 12 | 04_Storage_Network_Replay |
| src/storage/binary_storage.cpp | C++ | 42 | 04_Storage_Network_Replay |
| src/storage/binary_storage.h | C/C++ Header | 23 | 04_Storage_Network_Replay |
| src/storage/point_cloud_replay.cpp | C++ | 26 | 04_Storage_Network_Replay |
| src/storage/point_cloud_replay.h | C/C++ Header | 8 | 04_Storage_Network_Replay |
| src/storage/writer_interfaces.h | C/C++ Header | 21 | 04_Storage_Network_Replay |
| tests/acquisition_driver_test.cpp | C++ | 66 | 07_ROS_Tools_Deployment_Tests |
| tests/CMakeLists.txt | Text | 0 | 07_ROS_Tools_Deployment_Tests |
| tests/config_system_test.cpp | C++ | 12 | 07_ROS_Tools_Deployment_Tests |
| tests/core_contract_test.cpp | C++ | 5 | 07_ROS_Tools_Deployment_Tests |
| tests/detection_input_builder_test.cpp | C++ | 18 | 07_ROS_Tools_Deployment_Tests |
| tests/phase4_pipeline_test.cpp | C++ | 46 | 07_ROS_Tools_Deployment_Tests |
| tests/phase6_network_test.cpp | C++ | 4 | 07_ROS_Tools_Deployment_Tests |
| tests/phase7_cuda_performance_test.cpp | C++ | 3 | 07_ROS_Tools_Deployment_Tests |
| tests/phase7_performance_test.cpp | C++ | 3 | 07_ROS_Tools_Deployment_Tests |
| tests/phase7_realtime_qualification_test.cpp | C++ | 3 | 07_ROS_Tools_Deployment_Tests |
| tests/phase7_storage_performance_test.cpp | C++ | 2 | 07_ROS_Tools_Deployment_Tests |
| tests/phase7_storage_test.cpp | C++ | 27 | 07_ROS_Tools_Deployment_Tests |
| tests/point_cloud_postprocessor_test.cpp | C++ | 12 | 07_ROS_Tools_Deployment_Tests |
| tests/point_cloud_replay_test.cpp | C++ | 10 | 07_ROS_Tools_Deployment_Tests |
| tests/replay_setup_loader_test.cpp | C++ | 8 | 07_ROS_Tools_Deployment_Tests |
| tests/waymo_converter_test.py | Python | 2 | 07_ROS_Tools_Deployment_Tests |
| tools/convert_waymo_zip.py | Python | 7 | 07_ROS_Tools_Deployment_Tests |
| tools/export_waymo_frame.py | Python | 2 | 07_ROS_Tools_Deployment_Tests |